# ESA CryoVEx/ICESat-2 2019

Airborne and in situ field campaign - Data Acquisition Report

H. Skourup1, A. Di Bella1, A. Coccia2, A. Shepherd3, A. Hogg3, S. B. Simonsen1,

I. Otosaka3, A. Lemos3, F. Rodriguez-Morales4, R. Forsberg1 and T. Casal5

1 DTU Space, Technical University of Denmark, Kgs. Lyngby, Denmark

2 MetaSensing BV, Noordwijk, The Netherlands

3 CPOM, University of Leeds, LS2 9JT, UK

4 CRESIS, University of Kansas, USA

5 ESA ESTEC, NL

July, 2019

ESA contract report

Version: 1.0

Contract number:  4000120131/17/NL/FF/mg (CCN1)

Contractor: DTU Space

Authors: H. Skourup, A. Di Bella, A. Coccia, R. Forsberg and T. Casal

ESA technical officer: Tânia Casal

Front page:  Norlandair Twin Otters to support the airborne and in situ work. Credits: EU team.

- Contents

1	Introduction	4

2	Summary of operation	5

2.1	Day to day	8

3	Hardware installation	9

4	Overview of acquired data	13

5	Data handling	16

5.1	GPS and INS data	16

5.2	Airborne Laser Scanner (ALS)	17

5.3	ASIRAS	18

5.4	KAREN	18

5.5	Calibration and absolute heights of ASIRAS and KAREN	22

5.6	Camera	23

6	The EGIG line	24

6.1	In situ observations	24

7	Priority flights	25

8	Conclusion	27

9	References	28

10	APPENDIX Operator logs 	29

11	APPENDIX Overview acquired ALS data	35

12	APPENDIX Overview of acquired ASIRAS data	36

13	APPENDIX Overview of acquired KAREN data	37

# Introduction

The ESA CryoVEx/ICESat-2 campaign 2019 is the first CryoVEx campaign since the launch of NASA ICESat-2 in September 2019, and aims at cross-validating ESA CryoSat-2 and NASA ICESat-2 missions over sea ice and land ice in the Arctic. The campaign also extends the observations of dual-frequency (Ka/Ku-band) airborne observations, first flown in CryoVEx/KAREN 2016 fall campaign, to exploit the concept for future polar satellite missions. The airborne observations were coordinated with large scale in situ work along the EGIG line of the Greenland Ice Sheet.

The campaign involved operations with ESA’s Ku-band radar (ASIRAS), Dutch Company MetaSensing’s Ka-band radar (KAREN) and laser scanner using chartered Twin Otter (TF-POF) from Norlandair, Iceland. Three sea ice flights were flown in the Arctic Ocean out of Station Nord including 2 near coincident underflights of ICESat-2 and 1 of CryoSat-2, see Figure 1. A direct underflight of CryoSat- 2 was flown in Baffin Bay on route from Thule AB to Ilulissat. Transit flights crossing the Greenland Ice Sheet provides cross-overs of all altimeter missions (CryoSat-2, ICESat-2, Sentinel-3 and SARAL/AltiKa).

Unfortunately a planned coincident flight with Alfred Wegener Institute for Polar and Marine Research (AWI) airborne program called “IceBird” carrying a snow radar for independent measurements was not possible due to bad weather in CFS Alert. For similar reasons, a dedicated flight following ground tracks of CryoSat-2 and ICESat-2 over the Greenland ice sheet, was not possible.

This report outlines the airborne and in situ field operations conducted during March 23 – April 4, 2019. The campaign was coordinated by National Space Institute, Technical University of Denmark (DTU Space) in collaboration with MetaSensing. In addition to Danish/Dutch airborne team, ground validation was carried out by scientists from the University of Leads (UK), University of Kansas (US), and DTU Space.

The primary purpose of the project is:

Cross-validation of ESA CryoSat-2 and NASA ICESat-2 missions over land- and sea ice

To fly dual frequency (Ka/Ku-band) radar altimeters together with laser to study penetration depths in support of future satellite missions

Coordinated flight with extended in situ work on land ice (EGIG-line)

Figure 1: Overview of the flight tracks from the CryoVEx/ICESat-2 2019 airborne campaign. Yellow stars marks satellite passage time on the day of flight and blue snowflakes mark the in situ sites.

# Summary of operation

The CryoVEx/ICESat- 2 2019 airborne campaign, basically circumnavigated Greenland north of 70°N, see flight lines in Figure 1, with about 30 flight hours.

The campaign took place March 23 – April 4, 2019. The Norlandair Twin Otter (reg: TF-POF), which is the same aircraft as used throughout previous CryoVEx campaigns, was chartered. The instrument installation and test flights took place in Akureyri, Iceland, March 21-23, following the general instrument certification for the aircraft obtained in 2006 (Hvidegaard and Stenseng, 2006). Unfortunately the weather was not favorable, and included quite a few weather days where it was not possible to fly. Station Nord was experiencing snow and white out, whereas weather days in Ilulissat was due to heavy cross-winds on the runway, and rain followed by freezing.

The coincident flight with AWI Polar-6 was originally planned out of Station Nord. Due to restrictions flying directly from Svalbard to Station Nord, this had to be cancelled in advance. As an alternative the team tried to do a coincident underflight of ICESat-2 while Polar-6 was based in CFS Alert and CryoVEx team was based in Station Nord. Due to weather it was not succeeded.

Luckily, the weather was favorable and all planned flights were achieved within the estimated time.

The flight altitude during survey is typically 300 m agl, limited by the range of the laser scanner, and the nominal ground speed is 135 knots. The aircraft is equipped with an extra ferry tank permitting longer flights (5-6 hrs), and an autopilot for better navigation accuracy. In good conditions the across-track accuracy is down to a few meters using a custom-made navigation system connected to geodetic GPS receivers. Calibration flights of the instruments over buildings and runways were performed whenever possible. For a more detailed description see Section 5.3 and 5.4.

The airborne science team consisted of Henriette Skourup (HSK), Alessandro Di Bella (ADIA) from DTU Space, Alex Coccia (AC) from MetaSensing, and Tânia Casal (ESA). The in situ team consisted of Andy Shepard, Anna Hoggs, Ines Otosaka,  … Adriano, Fernando, Sebastian B. Simonsen.

An overview of the flights is found in Table 1 and 2 along with a detailed “day-to-day”-report in Section 2.1. Operator logs and detailed plots of flight tracks are provided in Appendix 10.

Table 1: Overview of CryoVEx/ICESat-2 2019 flights

| Date | DOY | Flight | Track | Take off UTC | Landing | Airborne | Airborne | Survey operator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UTC |  | accumulated [dd:hh:mm] |  |
| 23-03-2019 | 82 | a | AEY test flight | 13:10 | 13:50 | 00:40 | 00:00:40 | HSK/ADB/AC |
| 23-03-2019 | 82 | b | AEY-CNP | 15:06 | 17:10 | 02:04 | 00:02:44 | No Survey |
| 23-03-2019 | 82 | c | CNP-DMH | 17:38 | 20:18 | 02:40 | 00:05:24 | No Survey |
| 24-03-2019 | 83 | a | DMH-STN | 15:39 | 18:56 | 03:17 | 00:08:41 | HSK/ADB/AC |
| 25-03-2019 | 84 | a | STN-ICESAT(1327)-F2-F1-CAL-STN | 08:06 | 13:35 | 05:29 | 00:14:10 | HSK/ADB/AC |
| 29-03-2019 | 88 |  | STN-ICESAT(1343)-STN | 16:10 | 21:25 | 05:15 | 00:19:25 | HSK/ADB/AC |
| 30-03-2019 | 89 | a | STN-CS2()-IS2(16)-STN | 08:10 | 10:24 | 02:14 | 00:21:39 | HSK/ADB/AC |
| 30-03-2019 | 89 | b | STN-HAGF2-NORT6-NORT1-TAB | 11:03 | 16:04 | 05:01 | 01:02:40 | HSK/ADB/AC |
| 01-04-2019 | 91 | a | TAB-CS2(47570)-JAV | 12:45 | 17:15 | 04:30 | 01:07:10 | HSK/ADB/AC |
| 04-04-2019 | 94 | a | JAV-EGIG-CNP | 16:13 | 20:58 | 04:45 | 01:11:55 | HSK/ADB/AC |
| 04-04-2019 | 94 | b | CNP-AEY |  |  |  |  | No Survey |
| Total |  |  |  |  |  |  | 30h 28min |  |

## Day to day

The airborne part of CryoVEx/ICESat-2 2019 campaign progressed as follows:

March 20 	HSK, ADIA Copenhagen -> Akureyri

March 21-22 	Installation, Norlandair Hangar. Power rack cannot start up using 220V only aircraft power 28V. Prep new metal …. Tânia stuck in Rekjavik

March 23 	Tânia arrives in the morning. Test flight at noon. Akureyri -> Danmarkshavn via Constable Pynt. Too low clouds to measure Walterhausen glacier and ice sheet

March 24 	Standby in the morning. ETD 15.00 Danmarkshavn to Station Nord. Directly west to intercept line WALTH2 – NE2. Measure on the ice sheet to Station Nord. Some drift on the surface.

March 25 	Clear sky, -32C, calm. Flight ICESat-2 # 1327 almost on the line outside STN at the IS2 passage time. Measure northern (F2-F1) and eastern part (F1-STN) of triangle. AWI Polar-6 arrive at CFS Alert.

March 26-27 	grounded due to snow and low visibility. Polar-6 grounded due to weather. The icebridge in Nares Strait collapsed and open water outside the station.

March 28 	-30C, no snow, … Weather improved STN. Predictions area for planned coincident flight with AWI looks bad with snow and low clouds. AWI next … cancelled the flight of the day

March 29  	Delaying flight to 1600 UTC to wait for Polar-6. Flight along ICESat-2 track #1343 … AWI had to cancel due to weather.

March 30 	Clear sky, -30C at STN. Flying a short flight following CS2 #47570 and IS2 #16 out of station nord. Passage times outside the … TAB open within flight hours of Operation Northern Falcon (Ilusion) 1100-1700 UTC. Flight STN to TAB following Hagen Glacier and NORT6-1. Arriving TAB. (Opening of TAB 1.600 USD/hour). In situ team at T35. No heating of the iMAR 110V, TO in hangar.

March 31 	Day off due to closure of the airport. In situ team survey T9 and T12.

April 1 	TAB to JAV following CS2 track #47601. Change power supply for screen to pilots. Were at position … by the passage time of CS2 (14:48 UTC). Skipped Disko Island flight due to time and low clouds in the region. The afternoon flight planned for the EGIG line was cancelled due to heavy cross-winds at the runway (already marginal for the landing)

April 2-3 		No flying due to weather. April 2 windy, rain and +5C  - risk of icing. April 3 snow and windy.

April 4 		Snow and wind in the morning. EGIG line T1-T41 in the afternoon in clear conditions at the ice sheet. In situ team at T21 with corner reflectors. Five overflights of the site en route CNP. Arrive at midnight local. Deinstall the instruments 4 hours.

April 5 	SSIM, ADIA and HSK AEY-CPH

# Hardware installation

The hardware installation in the Norlandair Twin Otter (TF‐POF) consisted of the following instruments:

- MetaSensing Ka-band radar altimeter KAREN

- ESA Ku‐band interferometric radar ASIRAS

- DTU Space Airborne Laser Scanner (ALS) of the type Riegl LMS Q‐240i‐60

- Three geodetic dual‐frequency GPS receivers of type Javad Delta (AIR1-4), where AIR4/AIR3 was used to support ASIRAS time tagging

- An Inertial navigation system (INS) of the type Honeywell H‐764G

- An Inertial navigation system (INS) of the type iMAR

- An integrated NovaTel GPS-INS system to support KAREN

- Cameras (GoPro3 and GoPro7) for vertical and slant looking images

The KAREN sensor was for the first time successfully tested during CryoVEx/KAREN 2016 fall campaign (Skourup et al., 2017a). Since then, the horn antennas, have been replaced with new Microstrip patch array antennas, see Figure 2.

Figure 2: KAREN Radio Frequency (RF) enclosure with updated microstrip patch antennas.

The installation was similar to CryoVEx/KAREN 2017 campaign (Skourup et al., 2019). To avoid shadow effects from the hull of the aircraft the ALS was tilted slightly backwards. The Ka-band altimeter was fitted into the rear ASIRAS rack. A dedicated GPS-INS is used by the Ka-band altimeter for post processing, and it is installed in the rack together with the control unit.

The installation of the ASIRAS system was identical to the setup used throughout the previous CryoVEx campaigns (e.g. Skourup et al., 2013). Due to problems with PC2 during previous campaigns, ASIRAS had been modified before the campaign to rely on only PC1, leaving it with only option to measure in Low Altitude Mode (LAM). A Javad Delta receiver AIR4/AIR3 was used to support ASIRAS. The bottom view of the aircraft is shown in Figure 4, with the external ASIRAS antennas at Ku band, and the KAREN sensor at Ka band contained in the hatch.

Three geodetic dual-frequency GPS receivers (AIR1, AIR2 and AIR3/AIR4), mounted to log precise aircraft positioning, were connected to two separate GPS antennas (“front” and “rear”) through antenna beam splitters. The GPS antennas are permanently installed on TF-POF. Information about the antenna constellation is provided below:

Front antenna:

AIR2

AIR3 from 24-03-2019

AIR4 until 23-03-2019

- Rear antenna:

AIR1

The logging rates of the AIR2 and AIR3 were 1 Hz, whereas AIR1 and AIR4 (has not been downloaded as it broke) was logging at 2 Hz. The higher logging rate for AIR1 was chosen to obtain a higher precision for the on-board navigation system. AIR4 was also used to support ASIRAS time tagging. Offsets between GPS antennas and ASIRAS/ALS reference points are given in Table 2.

To record the attitude (pitch, roll and heading) of the aircraft, two inertial navigation systems (INS) were used. The primary unit is a medium grade INS of type Honeywell H-764G. This unit collects data both in a free-inertial and a GPS-aided mode at 50 Hz. Specified accuracy levels in roll and pitch are better than 0.1˚, and usual accuracy is higher than this. A new super-precise INS strap-down unit purchased from iMAR, which has the potential to measure the gravity field, was installed in the cabin next to the front operator seat, see Figure 3. The Honeywell INS was connected to the rear antenna. The iMAR was connected to the front GPS antenna. The setup of the instruments in the aircraft is shown in Figure 3 and pictures of the various instruments are shown in Figure 4-7.

Table 2: The dx, dy and dz offsets for the lever arm from the GPS antennas to the origin of the laser scanner, the back centre of the ASIRAS antenna (see arrow Figure 3), and the KAREN reference point.

| To laser scanner | dx (m) | dy (m) | dz (m) |
| --- | --- | --- | --- |
| from front GPS antenna | - 3.60 | + 0.49 | + 1.58 |
| from front GPS antenna | + 0.10 | - 0.38 | + 1.42 |
| to ASIRAS antenna | dx (m) | dy (m) | dz (m) |
| from front GPS antenna | -3.37 | +0.47 | +2.005 |
| from rear GPS antenna | +0.33 | -0.40 | +1.845 |
| to KAREN reference point | dx (m) | dy (m) | dz (m) |
| from front GPS antenna | -3.71 | +0.47 | +1.82 |
| from rear GPS antenna | -0.01 | -0.40 | +1.66 |

Reference point for antenna offset measurements

y

x

The Ka-band radar antenna, are mounted alongside the laser scanner.

The Ka-band radar system is added to the rear-ASIRAS rack

Figure 3: Overview of instrument setup in the TF-POF Twin Otter aircraft

Figure 4: Bottom view of the Norlandair Twin-Otter TF-POF showing KAREN and ASIRAS antennas together with ALS and GoPro camera.

| Figure 5: View of cabin in aircraft; Left picture: Rack with Ka-band altimeter (front right), rack for ALS, GPS and INS (rear left). Right picture: iMAR strap-down INS, grey box attached to the floor (lower right). |  |
| --- | --- |

Figure 6: The mount-plate seen from above. The ALS is seen in the lower part of the image just below the Ka-band RF module. The EGI is seen in the upper left corner.

Figure 7: Detail of the KAREN sensor: the new patch antennas allow for a compact installation. Part of the ALS is seen below the KAREN sensor as a bluish window and the GoPro camera to the left. Photo: A. Coccia.

# Overview of acquired data

Data from the various instruments were acquired where feasible, considering the limited height range of the ALS system and the weather. An overview of all acquired data is listed in Table 3.

The sampling frequency of the KAREN sensor was set to 25 MHz corresponding to one sampling each ~30cm on the ground. The high sampling frequency results in a large amount of 360 GB of raw data per hour, plus a few MB of navigation data (dedicated GPS/IMU module). Acquisitions have been manually started and stopped by the operator according to the area which was flown; data sets of typically ~10 minutes or ~20 minutes duration have been logged. At the end of the campaign the raw data amount is ~6 TB. During the flight and at the end of each acquisition day a quick data look was performed on randomly selected dataset to assess the quality and eventually adjusting the Pulse Repetition Frequency (PRF) according to the flight altitude.

As described in Section 3, ASIRAS could only measure in in Low Altitude Mode (LAM), due to the decoupling of PC2. This allows flight at an altitude of 300 m, which is within the operational range of the ALS system and a relative low data volume of about 28 GB per hour. A total of 567 GB raw ASIRAS data were collected during the CryoVEx/ICESat-2 2019 campaign. The data were stored on hard discs as ASIRAS level 0 raw data in the modified compressed format (Cullen, 2010).

In general, the ALS worked well. During all flights the last pulse option was used, to avoid internal reflections of the aircraft fuselage due to the limited space. The data volume obtained by the ALS is about 250-300 MB per hour, which is a relative small amount, when compared to the ASIRAS data volume. During the campaign a total of 32.8 GB ALS data was acquired.

The airborne GPS units logged data internally in the receivers during flight, which were downloaded upon landing on laptop PCs. The GPS reference stations listed in Tables 3 are described in further detail in Section 5.1.

Despite the limited space in the instrument bay, it possible to mount a nadir-looking GoPro camera next to the KAREN sensor, however with limited field of view. Additional slant looking GoPro was mounted on a rear window in the cabin. These images were only acquired for sea ice flights. For a more detailed description of images, see Section 5.5.

Table 3: Data acquisition overview from CryoVEx/ICESat-2 2019 campaign

| Date | DOY | AIR1 | AIR2 | AIR3 | AIR4 | EGI H-764G | iMAR | ALS | GPS REF1 | KAREN | ASIRAS | Nadir GoPro | Sidelooking GoPro | Log | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-03-2019 | 82a | X | X | - | X | X1 |  | X | X | X |  |  |  | X |  |
| 23-03-2019 | 82b | X |  | - | X | - |  |  | - |  |  |  |  | X | No Survey |
| 23-03-2019 | 82c | X | X | - |  | X2 |  |  | - |  |  |  |  | X | No Survey |
| 24-03-2019 | 83 | X | X |  |  | X2 |  | X | - | X | X |  |  | X |  |
| 25-03-2019 | 84 | X | X | X | - | X3 |  | X | X4 | X | X |  |  | X |  |
| 29-03-2019 | 88 | X | X | X | - | X |  | X | X | X | X |  |  | X |  |
| 30-03-2019 | 89a | X | X | X | - | X |  | X | X | X | X |  |  | X |  |
| 30-03-2019 | 89b | X5 | X | X | - | X |  | X | - | X | X |  |  | X |  |
| 01-04-2019 | 91 | X | X | X | - | X |  | X | - | X | X |  |  | X |  |
| 04-04-2019 | 94a | X |  | X | - | X2 |  | X | - | X | X |  |  |  |  |
| 04-04-2019 | 94b |  |  | - | - |  |  |  |  |  |  |  |  |  | No Survey |

1) Stopped before on ground, probably due to loss of satellites, as the rear antenna cable was out of order

2) In DASIN(dx) or ASIN(dx) or DACOS(dx) or ACOS(dx), DABS(dx).gt.1.0 (dx=-0.1683593620398824d+01).  Error occurs at or near line 239 of _MAIN__

3) Screen froze, restart EGI at the beginning of the flight (GAP 8.1741972-8.2612325)

4) Stopped before end of flight at 11:44:59 UTC (On ground 13:35:00)

5) Started 11:15:07.0000 take off 11:03 ?!?

AIR4 was not downloaded as it broke

# Data handling

The data processing is shared between MetaSensing (MS), the Alfred Wegener Institute (AWI) and DTU Space. Both KAREN and ASIRAS data are processed using GPS and INS data supplied by DTU Space to ensure a consistent baseline, which is possible as all the instruments are flown on the same platform. GPS differential positioning and combined INS-GPS integration is performed at DTU Space followed by processing of laser distance measurement into elevation above a reference ellipsoid. The KAREN data is primarily processed by MS with input and expert knowledge from AWI. The ASIRAS data was processed by AWI using standard procedures.

## GPS and INS data

The exact position of the aircraft is found from kinematic solutions of the GPS data obtained by the GPS receivers installed in the aircraft, see Chapter 3. Two methods can be used for post-processing of GPS data, differential (DIF) processing and precise point positioning (PPP). Whereas the first method uses information from reference stations in the processing procedure, the PPP method is only based on precise information of satellite clock and orbit errors.

A Javad Maxor Receiver (REF1) with internal antenna and logging rate 1 Hz were used as base station. The base station was mounted on DTU Space small tripods (vertical height 12 cm). However, the reference points were generally not marked, and thus the reference stations were not placed at the exact same position for the different flights, and a reference point has to be calculated for each flight. The post-processing of GPS data for both campaigns has just been finalized at DTU Space.

The position and attitude information (pitch, roll and heading) is measured with the Honeywell (H-764G) and iMAR inertial navigation systems. A dedicated GPS-INS is used by the Ka-band altimeter for post-processing.

## Airborne Laser Scanner (ALS)

The RIEGL LMS Q‐240i‐60 laser scanner uses 4 rotating mirrors, which results in parallel scan lines on the surface with a maximum scan-angle of 60˚.  The ALS operates with wavelength 904 nm, which is expected to reflect on the air-snow surface. The pulse repetition frequency is 10,000 Hz and the ALS scans 40 lines per second, thus the data rate is 251 pulses per line. This corresponds to a horizontal resolution of 0.7 m x 0.7 m at a flight height of 300 m and a ground speed of 250 kph. The across-track swath width is roughly equal to the flight height, and the vertical accuracy is in the order of 10 cm depending primarily on uncertainties in the kinematic GPS-solutions.

The raw logged files with start /stop times are listed in Appendix 11. The ALS data were pre-processed during the campaign to ensure a high quality and to prevent any problems with the instruments.

Calibration of ALS misalignment angles between ALS and INS can be estimated from successive overflights from different directions of the same building, where the position of the corners is known with high precision from GPS measurements. For this purpose buildings in Akureyri and at Station Nord have been measured, see example of overflight of calibration building (called Ebbes Koldhal) in Figure 8. Calibration maneuvers were carried out, as listed below:

•	23-03-2019 DOY 82	Akureyri, Iceland

•	25-03-2019 DOY 84 	Station Nord, Greenland

Figure 8: Map of the calibration building and runway at Station Nord measured by ALS on March 25, 2019

## ASIRAS

The ASIRAS radar operates at 13.5 GHz with footprint size 10 m across-track and 3 m along-track at a standard flight height of 300 m. The range resolution for ASIRAS is 0.1098m. ASIRAS specifications are provided in Table 4.

An overview of the acquired ASIRAS log-files together with start/stop times, range window and number of pulses are listed in Appendix 12.  The raw data has been shipped to AWI for further processing, and GPS/INS data solutions for priority flights March 25 and April 4 (Section 7) are ready and provided to AWI.

Table 4: Specifications of ASIRAS and KAREN at 300m flight altitude above the surface

| Instrument | Frequecy (GHz) | Footprint size |  | Range resolution (m) |
| --- | --- | --- | --- | --- |
|  |  | Along-track (m) | Across-track (m) |  |
| ASIRAS | 13.5 | 3 | 10 | 0.1098 |
| KAREN | 34.525 | 5 | 12 | 0.1650 |

## KAREN

The KAREN airborne instrument is an interferometric frequency modulated continuous wave (FMCW) SAR altimeter working at the Ka-band with central frequency of 34.525 GHz. The sensor operates with FMCW radar modulation in interferometric mode (SARIn), with one transmitting antenna and two receiving antennas. Radiation characteristics of the patch antennas include a gain of 22 dBi and a 3dB aperture of 15° x 4.6° (along track and cross track).  The processed multi-looked data corresponds to an along- and across-track footprint size of 5 m (100 looks) and 12 m, when flying at 300 m AGL, respectively. The vertical resolution is 0.1650 m when transmitting 600 MHz of bandwidth (tunable).  KAREN specifics are provided in Table 4.

An overview of the acquired data is provided in Appendix 13. The name of each file represents the date and starting time of the acquisition. For each acquisition the file size and some additional notes are given, such as duration of acquisition, altitude of the aircraft during the acquisition and area of interest.

Figure 9 shows some plots generated in-flight during acquisitions for quick-data analysis purposes: raw data, Range Doppler maps and backscattering profiles, for both receiving channels, together with a coherency map are given.

During the campaign, an anomaly has been found on one of the two sampling channels. Within each acquisition after some time one ADC randomly introduced some “out of range” samples (see Figure 10). These were the cause for some deteriorating quality of the data (see Figure 11).

The issue has been solved in the processing phase, by introducing an extra step with respect to the standard processing chain. The aim is to “filter out” the samples out of range in the faulty channel and substituting them with an interpolated value. A comparison analysis on the data collected by the two separate channels showed good results after the correction step has been implemented (see Figure 12).

In Figure 13 an example of KAREN processed profile is given as a reference, acquired on the 25th of March 2019.

Figure 9: Plots generated during flight for quick check

Figure 10: Example of misbehavior of the ADC during one acquisition: out of range samples are arbitrarily introduced in the raw data of channel 2.

Figure 11: Example of RD maps processed for the two RX channels of KAREN. Top: channel 1 showing the expected behavior. Bottom: channel 2, where the ADC issue was present, showing much higher noise level.

Figure 12: Example of RD maps processed for the two RX channels of KAREN after the filtering of ch2 data. Top: channel 1 Bottom channel 2, both showing the expected behavior.

Figure 13: Example of processed waveform from KAREN data, namely KAR_OPER_Level1b_20190325T122005_20190325T124149_levc.nc.

## Calibration and absolute heights of ASIRAS and KAREN

To obtain absolute surface heights from ASIRAS and KAREN an offset needs to be applied to account for internal delays in cables and electronics, see Sections 5.4.1 and 5.5.1. As the offset is dependent on the choice of retracker it has not been applied in the final Level 1b processing.  The offset is estimated by comparing ASIRAS and KAREN surface heights to surface heights obtained by ALS over a surface, where both the radar and the laser are known to reflect at the same surface.  Such measurements are typically obtained by overflights of runways. Different biases apply for the different aircraft installations, see Section 3.

The runway overflights performed during CryoVEx/ICESat-2 2019 campaign is listed below:

- 25-03-2019 DOY 84 	Station Nord

- 04-04-2019 DOY 94 	Ilulissat, Greenland

## Camera

To complement the analysis of KAREN, ASIRAS and ALS data over sea ice high resolution images are collected along the flights. A nadir looking GoPro3 camera was installed in the baggage compartment with limited field of view due to the limited space. A slant looking images were obtained using a GoPro7 camera. The camera was mounted in the rear starboard window in the cabin. During ESA CIMR campaign in March it was found that GoPro7 was more prone to cold temperatures. Both cameras were remote controlled and time tagged using the internal camera clock. By combining the time tag of the images with GPS data the images can be geo-located along the flight lines. An overview of the properties of the cameras is given in Table 5 and examples are shown in Figure 14.

GoPro3, nadir photos

23-30/03/2019 → Pics time = UTC+1

1-4/04/2019 → Pics time = UTC+2

GoPro7, off-nadir photos (right side w.r.t. flight direction)

Pics time = UTC

Table 5: Overview of camera types and settings.

| Camera type | View | Interval (sec) | Resolution (pixels) | Image size (MB) | Software program | Format |
| --- | --- | --- | --- | --- | --- | --- |
| GoPro 3 | Nadir-looking | 5/2 | 2592x1944 | ~1.5 | Alessandro ? | JPEG |
| GoPro 7 | Slant-looking | 5 | 3000x4000 | ~2.2 | Alessandro ? | JPEG |

| Figure 14: Examples of nadir looking image from GoPro3 (left) and slant-looking image from GoPro7 (right) taking during flight on March 25, 2019. |  |
| --- | --- |

# The EGIG line

The CryoVEx April 2019 land ice field team visited 4 field sites along the EGIG line in Greenland. The overall aim was to sample different snow conditions on the Greenland ice sheet, namely the dry snow zone, the percolation zone, and the ablation zone. The science experiments conducted aim at informing how different snow conditions affect the altimeter echo. These in-situ measurements will help the validation and interpretation of airborne data collected by DTU Space and space-borne altimeters (CryoSat-2, IceSat-2, Sentinel-3 and AltiKa) flying over the EGIG line.

## In situ observations

A map of the field sites is shown in Figure 15 below.

Figure 15: Locations of Airports () and locations on the EGIG-line ().

The information listed below provides a general schedule for the land ice field campaign in Spring 2019 on the Greenland ice sheet (Table 6).

Table 6: Overall schedule of the 2019 campaign

| Date | Travel plan |
| --- | --- |
| 28/03/2019 | IO and AH Leeds to AEY |
| 29/03/2019 | Equipment preparation |
| 30/03/2019 | Flight AEY-CNP-T35-JAV, AS Brussels to JAV, AL CPH to JAV |
| 31/03/2019 | Flight JAV–T12–T9-JAV |
| 01/04/2019 | Bad weather in JAV |
| 02/04/2019 | Bad weather in JAV, AS returns to Leeds |
| 03/04/2019 | Bad weather in JAV |
| 04/04/2019 | Flight JAV-T21-CNP-AEY, IO and AH return to Leeds |

The following locations (Table 7) were visited during the field campaign and the experiments conducted at each site are listed in Table 8.

Table 7: Overview of CryoVeX 2019 EGIG site locations

| Site | Date | Base station start (UTC) | Base Station stop (UTC) | Latitude [deg. N] | Longitude [deg. W] |
| --- | --- | --- | --- | --- | --- |
| T9 | 31/03/2019 | 13:49 | 15:35 | 70 01.144N | 46 18.405W |
| T12 | 31/03/2019 | 11:54 | 13:30 | 70 10.419N | 45 20.761W |
| T21 | 04/04/2019 | 15:00 | 16:20 | 70 32.611N | 43 01.519W |
| T35 | 30/03/2019 | 16:15 | 18:00 | 70 57.03N | 39 21.72W |

Table 8: Overview of acquired data at the T-sites

| Date | Site | GPS | Core | Radar | Magnaprobe | Overflight reflectors |
| --- | --- | --- | --- | --- | --- | --- |
| March 30th | T35 | X | X | X | X | - |
| March 31st | T12 | X | X | X | X | - |
| March 31st | T9 | X | X | X | X | - |
| April 4th | T21 | X | X | X | X | X |

- Firn cores

Four shallow firn cores to directly measure the density of the top ~3 metres of snow were collected during the 2019 Spring campaign.

A coring system with a drill barrel of 9 cm diameter was used to extract ice cores down to ~6 m to obtain snow density and stratigraphy. Once extracted, the ice cores were stored in an ice core box and were taken to Ilulissat where detailed stratigraphy analysis of the ice cores was performed by illuminating the ice cores and taking photographs to identify the snow and ice layers.

- Magnaprobe

To measure the depth of the first ice layers and investigate the spatial variability of the snow conditions around each sites, snow depth measurements were recorded using the magna probe lent by the Norwegian Polar Institute. The magna probe records the snow depth, GPS position and time of the measurement and can measure a depth up to 120 cm. An example of the data collected is shown on Figure 16.

Figure 16: Snow depth measurements recorded with the magna probe at T35

- Radar

Static radar measurements were acquired at each site at both Ka and Ku-band using a radar built by CReSIS and a kinematic test was carried out at T21 over ~1.2 km.

- Corner reflectors

Three corner reflectors (one for Karen and two for ASIRAS) made in aluminum by MetaSensing were laid out on the snow surface to act as a strong surface scattering horizon. Figure 17 below shows the setup of the corner reflectors.

Figure 17: Setup for the corner reflectors overflight

We set the metal plate corner reflectors out at T12 on 4th April 2019. The corner reflector for Ka-band was fixed on a tripod while the two Ku corner reflectors were laid out on the snow. The height above the snow surface at which the corner reflectors were placed is given in Table 9.

Table 9: Corner reflectors site information

| Number | Height above snow surface (cm) |
| --- | --- |
| 1 | -7 |
| 2 | +114 |
| 3 | +4.5 |

Figure 18: Ground team at work during acquisition over the EGIG line.

# Priority flights

Flights on acquired on the EGIG line flight on April 4 and ICESat-2 underflight on March 25 following ICESat-2 orbit #1327 cycle 2 has been given priority to be quickly processed. Both flights include runway overpasses (see Section 5.5). INS and GPS data have been delivered to support KAREN and ASIRAS processing, and first look of ALS data is provided in Figure 15 and 16.

| Figure 19: ALS elevations at T21 crossing corner reflectors several times (left) and ground team Twin Otter as seen by the nadir looking GoPro3. Acquired on April 4, 2019. |  |
| --- | --- |

Figure 20: ALS with respect to DTU15 MSS over sea ice following ICESat-2 orbit #1327 cycle 2, acquired on March 25, 2019.

# Conclusion

The CryoVEx/ICESat-2 2019 Arctic campaign has for the first time collected data along ICESat-2 ground tracks. Two near-coincident ICESat-2 tracks and two CryoSat-2 ground tracks over sea ice in the Wandel Sea north of Station Nord and in the Baffin Bay were underflown representing different sea ice types and settings.  The flights were coordinated with large scale in situ work along the EGIG line on the Greenland Ice Sheet.  Due to weather it was not possible to make direct underflights over the Greenland Ice Sheet, but the EGIG line is unique as it has several crossings of multiple altimetry missions including ICESat-2, CryoSat-2, Sentinel-3 and SARAL/AltiKa.

During the campaign data was acquired with MetaSensing’s Ka-band radar KAREN, ESA’s Ku-band radar (ASIRAS) and high-resolution near-infrared airborne Laser Scanner (ALS). Combined GPS and INS post-processed data has been delivered for the priority flights (March 25 and April 4) to support further processing of KAREN, ASIRAS and ALS. First results of post-processed ALS data are of high quality. The consequences of a small issue found during the campaign in one of the two sampling channels of the KAREN instrument have been minimized in the processing phase, at the end the data quality is not different for the two channels.

Despite several weather days the CryoVEx/ICESat-2 2019 campaign was successfully concluded and the data adds unique measurements to validate ICESat-2 and CryoSat-2. The results will assist on-going calibration and validation activities and support decision-making of future dual frequency satellite missions.

# References

Cullen, R.: CryoVEx Airborne Data Products Description, Issue 2.6.1, ESA, Ref. CS-LI-ESA-GS-0371, 2010

Hvidegaard, S. M., H. Skourup, J. Wilkinson, R. Ladkin, and R. Forsberg: ICE-ARC airborne campaign 2015 - Data collection and processing report, June 2017.

Hvidegaard, S. M. and L. Stenseng: Test of ASIRAS installation in Air Greenland Twin-Otter. Field Report, National Space Center, 2006

H. Skourup, S. B. Simonsen, A. Coccia, and R. Forsberg:  ESA CryoVEx/KAREN 2016 campaign - Airborne field campaign with Ku/Ka-band radar, and laser scanner. Data Acquisition Report, National Space Institute (DTU Space), Technical University of Denmark, February 2017

# APPENDIX Operator logs

DOY 082, March 23, 2019: Test flight, CAL AEY

ALS:

EGI aligned

1308	Start taxi

132855	Start scan file

1332	Building overflight N-> S

1335	Building overflight E-> W

1344	Building overflight N-> S

KAREN problems finding satellites

1350	On ground

ASIRAS:

Test on ground ASIRAS (no HPA)

1311 	Take off

iMAR looks OK

only GoPro3 at 2s interval

No ASIRAS

1352 	Landing

DOY 082, March 23, 2019: AEY-CNP, CNP-DMH

ALS:

1506	Take off AEY

No surveying

AIR2 no satellites

Switch AIR2/AIR4 on rear splitter

1710	On ground CNP

1738	Take off CNP

Running GPS, EGI and iMAR

Very low clouds Walterhausen Glacier – no go

2018	On ground DMH

ASIRAS:

No ASIRAS or

No GoPro

DOY 083, March 24, 2019: DMH-GrIS -STN

ALS:

Before flight changed rear GPS antenna ta splitter cable

AIR4 not working, power connector in GPS broken

AIR3 setup to support ASIRAS

iMAR not logging for 10 min before flight

1539	Take off DMH

1633	on the line (WALTH2-NE2)

163815	New scanner file

1640	ASIRAS logging

173509	New scanner file, set to last pulse TS1

180730	New scanner file, over land

1856	On ground STN

ASIRAS:

Start up with no ASIRAS as AIR4 supporting ASIRAS is not working properly.

Found a difference between PC1 and UTC time (ASIRAS UTC time = ASIRAS PC1 time + 1h 7m 56s). Below, UTC time is used

1535 	Take off

iMAR position LED always yellow/red, maybe not aligned for long enough? Or is it expected due to high latitudes?

Only GoPro3 (2s interval)

ASIRAS no calibration

1641		start A190324_00 (ASIRAS)

1739		start 01

1856	Landing

DOY 084, March 25, 2019: STN-IS2(1327c2)-F2-F1-CAL-STN

ALS:

Start up system using ground power

0806	Take off

New scanner file

Navigation system not ok

0810	ICESat-2 passage time

INS frozen – reboot, OK

0848	Navigation system ok

Suspect the power supply to be broken

0903	Some small refrozen leads

091015	New scanner file

0926	Poor visibility N84

Many leads

Strong headwinds

100909	New scanner file

1012	N85° 25 break track

Go to F2

1013	Teardrop

1020	F2

Low thin clouds

1033	Big lead with thin ice

1045	Huge area of thin ice

111100	New scanner file

1216	EOL F1

121615	New scanner file

1259	Crossing icedivide

Rubble field

1308	New scanner file

1316	Runway overflight N -> S

1319	Runway overflight S -> N

1324	Building overflight W-> E

1328	Building overflight S-> N

1335	On ground

ASIRAS:

Found a difference between PC1 and UTC time (ASIRAS UTC time = ASIRAS PC1 time + 1h 7m 58s). Below, UTC time is used

IS2+CS2 sea ice flight

0806		Take off

iMAR position led always yellow/red

Start GoPro3 at 5s interval

ASIRAS no calibration

0815		start A190325_00 (ASIRAS)

0918		start 01

1018		GoPro3 switched back to 2s interval

1018		GoPro7 turned on to 2s interval (found a way to mount it

inside)

1019		start 02

1121		start 03

1221		start 04 (end of record there should be the runaway

overflight)

1319		start 05 (short record, probably not useful)

iMAR files seem to be named  from "000" to "007",

instead of date and time, something with the GPS (ask 		Tim)

1335 	Landing

DOY 088, March 29, 2019: STN-IS2(1343c2)-STN

ALS:

1543	Turn on engine

Power off when switching from 1 -> 2 engines

Restart system

1604	Taxi

1610?	Take off Station Nord

No measurements on route STN-WP1

174200	New scanner file

1744	WP1

Scanner set to TS1

1747	New scanner file

1759	Transition zone

1801	WP2

1804	The first leads

1817	WP3, some haze

@N85 04.10 clear sky

1833	WP4

1852	WP5, 85N

Teardrop

Measuring WP5-STN

190000	New scanner file

193000	New scanner file

2020	85% battery blinking, “OK” not green

2026	New scanner file

2033	Rubble fields

2044	Land

2058	Fjord ice/sea ice

2125	On ground STN

Stop scanner

Battery dead before download finished

ASIRAS:

ASIRAS UTC time = ASIRAS PC1 time + 5s. Below, UTC time is used

1600 	Take off

iMAR position led always yellow/red

1739		Start both GoPros at 2s interval

1740		ASIRAS LAM calibration (_00)

1747		start A190329_00 (ASIRAS)

1855		start 01

1954		start 02

2054		start 03

2119 	stop recording

iMAR files back to be named date and time

?		Landing

DOY 089, March 30, 2019: STN-IS2(16c3)-CS2(47570)-STN

ALS:

Power system turned off even though ALS, heater and iMAR was switched off

0807	Taxi

0810? 	Take off

TS1

0819	New scanner file

On CryoSat line

0835	On line IS1-IS2

0844	IS2

0915	Break the line

091700	New scanner file

0920	Crossing line CS2-CS3

@ N83 38.29

0923	On line CS2-CS3

1001	Rubble fields

1015	CS3

1022	Stop scanner

1024	On ground STN

ASIRAS:

ASIRAS UTC time = ASIRAS PC1 time + 6s. Below, UTC time is used

0800 	Take off

iMAR position led always yellow/red

0821 	Start GoPro3 at 2s interval

0822	ASIRAS LAM calibration (_00)

0824	start A190330_00 (ASIRAS)

0828 	Start GoPro7 at 5s interval, internal GPS option enabled

0916 	turn back, tear drop towards second line

0923	start 01

0945 	iMAR message "status of IMU invalid has changed"  (x2)

0948 	PC1 crash, restarted (coincident with Alex switching off

Karen, although shouldn’t be related)

0951	start 02

1016 	stop recording

1017	Landing LAM calibration (_01)

1022 	stop GoPro3

1025 	stop GoPro7

1025 	Landing

1030 	GPS off right after landing, rack battery didn't work!

1030 	iMAR stop recording, as no GPS available

DOY 089, March 30, 2019: STN-HAGF2-NORT6-NORT1-TAB

ALS:

1054	System on

1101	Taxi

1103 	Take off

113800	New scanner file

1144	HAGF2

1207	NORT6

Teardrop

1210	NORT6 crossing

1219	NORT5 close to nunatak

1221	Land

123700	New scanner file

1306	NORT4

Drifting snow on surface

133900	New scanner file

1411	NORT3

1433	NORT2

144100	New scanner file

1507	85% Battery blinking on power rack

1522	NORT1

1529	Of the ice, stop survey

1604	On ground TAB

ASIRAS:

1105 	Take off

iMAR position led always yellow/red

No GoPros, land ice

1140		ASIRAS LAM calibration (_02)

1141		start A190330_03 (ASIRAS)

1141 	No GPS signal for ASIRAS, check software and cables,

ASIRAS and PC1 restarted (found out it was a faulty cable connection)

03 and 04 records not valid

1154		start 05

1256		start 06

1402		start 07

1508		start 08

1528		start 09 (very short and out of range window,

probably not valid)

1530		Landing LAM calibration (_03)

1531 	ASIRAS off

?		Landing

?		GPS off right after landing, pilot suddenly shut off power

and rack battery didn't work!

?		iMAR stop recording, as no GPS available

DOY 091, April 1, 2019: TAB-CS2(47601)-JAV

ALS:

1230	Engine on

1239	Taxi

1245	Take off

134915	New scanner file

1351	CS1

1418	First thin leads

1421	Frost flowers

1426	Huge area of thin ice

1439	CS2

144230	New scanner file

1448	CS2 passing @ N73° 16 W57° 52

1530	CS3

1534	Snow

1538	Climbing ~600m

1546	Descending ~300m

154800	New scanner file

1610	Break the line @ N70° 22

Right turn

1612	Crossing the line C3-C4

1613	Inbound JAV

1614	Stop surveying

1715	On ground JAV

ASIRAS:

ASIRAS UTC time = ASIRAS PC1 time + 8s. Below, UTC time is used

1220		Start iMAR logging, but PC went in stand-by (no aircraft power available and power rack battery down + could 		not get in the aircraft due to refueling operations)

1230		iMAR GPS on, logging should be good from here on

1245 	Take off

iMAR position led switches between green, yellow and red

1348		ASIRAS LAM calibration (_00)

1351		start A190401_00 (ASIRAS)

13:53		Start GoPro7 at 5s interval, internal GPS option enabled

13:54 	Start GoPro3 at 2s interval

14:54		start 01

15:30		start thick clouds

15:37		climbing to ~700m

15:54		start 02

16:09 	stop recording

16:10		Landing LAM calibration (_01)

16:24 	stop GoPro7

16:25 	stop GoPro3

17:15 	Landing

DOY 094, April 4, 2019: JAV-EGIG(T1-T41)-CNP

ALS:

In situ team on ground at T21, Corner reflectors 5 crossings

1550	Engine on

1620	Taxi

1613	Take off

161530	New scanner file

1628	Runway overflight SW-NE

On route T1

163730	New scanner file

1702	T1

Off the line by 2.5km

Aligning for T9

1720	T9, on the line

In situ team landed @T21

1729	T12

1736	T15

173800	New scanner file

1741	T17

1747	T19

1752	T21, in situ site

1755	Crossing CR

1759	Crossing CR

1804	Crossing CR

1809	Crossing CR, tail 2 nose

1813	Crossing CR

1827	T21 en route T35

1849	Speed increased to 150kn

1856	No GPS signal rear antenna

Loose connection in splitter

1858	GPS signal rear splitter OK

1859	T35

A bit off track, aligning

1911	T41

Stop surveying

On ground

ASIRAS:

ASIRAS UTC time = ASIRAS PC1 time + 15s. Below, UTC time is used

1600		start iMAR logging

1610 	Take off

iMAR position led mostly green

No ASIRAS calibration

1615		start A190404_00 (ASIRAS)

1628 	stop ASIRAS (local flight)

1632		start 01 (transition water/ice)

1635-41	sudden steep topography variations (out of range window several times + 2s of saturation)

1642		Karen starts logging

1729		start 02

1752		start maneuvers to align for overflight

1752 	Start GoPro3 at 0.5s interval (T21, ground team overflight in G0041880-1917)

1803		Start GoPro7 video (T21, got a tiny piece of NLC and ground team, ~25s)

Second pass, manual video (got a tiny piece of NLC and ground team, ~40s)

?		overflight is ~4km off track, check record 02. Aircraft turned back to survey again

1827		start 03 (right before getting on the right track for overflight)

1913		Landing LAM calibration (_00)

?		Landing

No power at landing, only 5 minutes iMAR post-processing

DOY 094, April 4, 2019: CNP-AEY

No surveying

# APPENDIX Overview acquired ALS data

|  | DOY | File name | Start (dechr) | Stop (chr) | Comments |
| --- | --- | --- | --- | --- | --- |
| 18-03-2017 | 082 | 082_130800.2dd 082_132900.2dd | 13.13393 13.47513 | 13.34241 13.74312 | Calibration AEY |
| 18-03-2017 | 083 | 083_163815.2dd 083_173500.2dd 083_180730.2dd | 16.63725 17.58337 18.12506 | 17.56607 18.03125 18.52612 |  |
| 18-03-2017 | 084 | 084_081020.2dd 084_091115.2dd 084_100900.2dd 084_111100.2dd 084_121615.2dd 084_130800.2dd | 08.18467 09.17064 10.14981 11.18313 12.27060 13.13315 | 09.15750 10.12831 11.17075 12.26000 13.12330 13.48232 | Calibration STN |
| 18-03-2017 | 088 | 088_174200.2dd 088_174700.2dd 088_190030.2dd 088_193000.2dd 088_202600.2dd | 17.70006 17.78340 19.00819 19.50008 20.43347 | 17.74628 18.99878 19.48661 20.40989 21.42017 | No data |
| 18-03-2017 | 089 | 089_081930.2dd 089_091700.2dd 089_113800.2dd 089_123700.2dd 089_133900.2dd 089_144100.2dd | 08.32522 09.28340 11.63339 12.61676 13.65005 14.68342 | 09.27402 10.37259 12.60863 13.64059 14.67297 15.52387 |  |
| 18-03-2017 | 091 | 091_134915.2dd 091_144230.2dd 091_154800.2dd | 13.82088 14.70840 15.80007 | 14.69705 15.78888 16.23511 |  |
| 18-03-2017 | 094 | 094_161530.2dd 094_163730.2dd 094_173800.2dd | 16.25835 16.62507 17.63343 | 16.56138 17.62186 19.24944 | Runway JAV |

# APPENDIX Overview of acquired ASIRAS data

| Date | File name | Start time (UTC) | End time (UTC) | Range window (m) |  | # Pulses |
| --- | --- | --- | --- | --- | --- | --- |
| 23-03-2019 | A190323_00.log | 13:17:34 | 13:18:06 | 90.00 |  | 75000 |
| 24-03-2019 | A190324_00.log A190324_01.log | 16:41:53 17:40:35 | 17:40:35 18:00:37 | 90.00 90.00 |  | 8799942 2999980 |
| 25-03-2019 | A190325_00.log A190325_01.log A190325_02.log A190325_03.log A190325_04.log A190325_05.log | 08:15:29 09:17:08 10:18:59 11:21:51 12:21:07 13:19:13 | 09:17:08 10:18:59 11:21:50 12:21:07 13:19:12 13:21:59 | 90.00 90.00 90.00 90.00 90.00 90.00 |  | 9242443 9272439 9422439 8884942 8709943 412498 |
| 29-03-2019 | A190329_00.log A190329_01.log A190329_02.log A190329_03.log | 17:47:56 18:55:11 19:54:25 20:54:06 | 18:55:11 19:54:25 20:54:06 21:19:37 | 90.00 90.00 90.00 90.00 |  | 10084933 8879940 8947439 3822474 |
| 30-03-2019 | A190330_00.log A190330_01.log A190330_02.log A190330_03.log** A190330_04.log** A190330_05.log A190330_06.log A190330_07.log A190330_08.log A190330_09.log | 08:24:06 09:23:55 09:50:36 11:44:08 11:47:16 11:53:44 12:56:17 14:03:00 15:08:58 15:28:56 | 09:23:54 09:43:55* 10:15:56 11:47:16* 11:47:16* 12:56:16 14:02:59 15:08:58 15:27:54 15:29:04 |  | 90.00 90.00 90.00 90.00 90.00 90.00 90.00 90.00 90.00 90.00 | 8964945 ? 3797475 0 0 9374939 10002433 9892433 2834981 15000 |
| 01-04-2019 | A190401_00.log A190401_01.log A190401_02.log | 13:51:37 14:54:40 15:54:50 | 14:54:39 15:54:50 16:09:54 |  | 90.00 90.00 90.00 | 9449939 9022440 2254986 |
| 04-04-2019 | A190404_00.log A190404_01.log A190404_02.log A190404_03.log | 16:15:54 16:32:38 17:29:08 18:27:11 | 16:28:52 17:29:07 18:27:10 19:11:26 |  | 90.00 90.00 90.00 90.00 | 1942488 8467445 8702441 6634955 |

* measurement log file corrupted, time retrieved from file

** record corrupted, no available data

# APPENDIX Overview of acquired KAREN data

Table 10 - Acquisition overview of the 23rd of March 2019 over Akureyri.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190323133143 | 600 | Calibration tracks over the airport |
| 20190323133558 |  |  |
| 20190323134335 |  |  |

Table 11 - Acquisition overview of the 24th of March 2019, transfer flight to Station Nord.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190324164403 | 600 | Short |
| 20190324164609 |  | 10 mins |
| 20190324165706 |  | 10 mins |
| 20190324171040 |  | 20 mins |
| 20190324173457 |  | Short |
| 20190324173556 |  | 20 mins |
| 20190324175658 |  | 3 mins |

Table 12 - Acquisition overview of the 25th of March 2019, local flight over sea ice from Station Nord.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190325081555 | 600 |  |
| 20190325082323 |  |  |
| 20190325083531 |  | 10 mins |
| 20190325084603 |  | 10 mins |
| 20190325085627 |  | 5 mins |
| 20190325090113 |  | 30 mins |
| 20190325093228 |  | 30 mins |
| 20190325100411 |  | 8 mins |
| 20190325102531 |  | 20 mins |
| 20190325104635 |  | 10 mins |
| 20190325105733 |  | 33 mins |
| 20190325113140 |  | 20 mins |
| 20190325115426 |  |  |
| 20190325122005 |  | 20 mins |
| 20190325124937 |  | 17 mins |
| 20190325131551 |  | Calibration track over runway |

Table 13 - Acquisition overview of the 29h of March 2019, local flight over sea ice from Station Nord.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190329170349 | 300 | HAM mode at ~1700 m AGL |
| 20190329170614 | 200 |  |
| 20190329170930 | 200 |  |
| 20190329171401 | 300 |  |
| 20190329172505 | 150 |  |
| 20190329174802 | 600 | 15 mins |
| 20190329180406 |  | 20 mins |
| 20190329182434 |  | 20 mins |
| 20190329184443 |  | 8 mins |
| 20190329185521 |  | Short |
| 20190329185808 |  | Crossing line |
| 20190329190056 |  | 10 mins |
| 20190329191152 |  | 10 mins |
| 20190329193435 |  | 20 mins |
| 20190329200121 |  | 30 mins |
| 20190329203754 |  | 10 mins |
| 20190329205852 |  | 10 mins |

Table 14 - Acquisition overview of the 30th of March 2019, local flight over sea ice from Station Nord.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190330082709 | 600 | 5 mins |
| 20190330083206 |  | 30 mins |
| 20190330090441 |  | 12 mins |
| 20190330092421 |  | 10 mins |
| 20190330093425 |  | Short |
| 20190330094951 |  | 8 mins |
| 20190330095940 |  | 15 mins |

Table 15 - Acquisition overview of the 30th of March 2019, transfer flight to Thule AB.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190330114422 | 600 | 24 mins |
| 20190330121050 |  |  |
| 20190330121238 |  | 5 mins |
| 20190330121726 |  | Short |
| 20190330121951 |  | 33 mins |
| 20190330130011 |  | 5 mins |
| 20190330130714 |  | 30 mins |
| 20190330133814 |  | 10 mins |
| 20190330134851 |  | 20 mins |
| 20190330140847 |  | 12 mins |
| 20190330142019 |  | 20 mins |
| 20190330143058 |  | 10 mins |
| 20190330145253 |  | 20 mins |
| 20190330151424 |  | 10 mins |

Table 16 - Acquisition overview of the 1st of April 2019, transfer flight to Ilulissat.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190401135314 | 600 | 20 mins |
| 20190401141331 |  | 10 mins |
| 20190401142336 |  | 21 mins |
| 20190401144442 |  | 10 mins (satellite overpass) |
| 20190401145456 |  | 10 mins |
| 20190401150508 |  | 20 mins |
| 20190401152531 |  | 15 mins, AC climbing for weather conditions |
| 20190401154011 |  | Short |
| 20190401154332 |  | Short |
| 20190401154635 |  | 20 mins |
| 20190401160719 |  | Short |

Table 17 - Acquisition overview of the 4th of April 2019, EGIG line to Constable point.

| # | Bw [MHz] | Notes |
| --- | --- | --- |
| 20190404161623 | 600 |  |
| 20190404161949 |  | Satellite overpass |
| 20190404163219 |  |  |
| 20190404164359 |  | 10 mins |
| 20190404165949 |  | 10 mins – T1 |
| 20190404171004 |  | 3 mins |
| 20190404171259 |  | T9 - 10 mins |
| 20190404172323 |  | 5 mins |
| 20190404172843 |  | 20 mins – T12 |
| 20190404174840 |  |  |
| 20190404175505 |  | Passes over survey team on ground |
| 20190404575919 |  |  |
| 20190404180347 |  |  |
| 20190404180852 |  |  |
| 20190404181230 |  |  |
| 20190404181321 |  |  |
| 20190404182726 |  | 30 mins |
| 20190404190039 |  | Short |
| 20190404190439 |  | Short |